Table of Contents

# HU University Network — Full Device Configuration Guide

### Project 2: New Network Design for Two University Buildings (Group-2)

## 0. IMPORTANT DESIGN CLARIFICATION — READ FIRST

Before the configs, one correction to keep the whole design consistent:

- VLAN gateways (SVIs) live on the DISTRIBUTION switches, not the Core switches.

- Distribution ↔ Access links = TRUNK (carry all VLANs, tagged with 802.1Q).

- Core ↔ Distribution links = ROUTED Layer-3 links (this is why we gave them /30 point-to-point subnets earlier — trunks never get a single IP, only routed links do).

- Core switches are pure Layer-3 forwarders: they route between Distribution, Firewall, Server, and WLC using OSPF.

This matches everything we already built in the VLSM table — nothing changes there, this is just making explicit where each config command belongs.

## 1. Quick Reference — VLANs

| VLAN ID | Name | Subnet | Gateway IP (on Distribution SVI) |
| --- | --- | --- | --- |
| 10 | INFORMATICS | 172.16.0.0/26 | 172.16.0.1 |
| 20 | ENGINEERING | 172.16.0.64/26 | 172.16.0.65 |
| 30 | LIBRARY | 172.16.0.128/26 | 172.16.0.129 |
| 40 | ADMIN | 172.16.0.192/26 | 172.16.0.193 |
| 50 | STAFF | 172.16.1.64/26 | 172.16.1.65 |
| 99 | MGMT | 172.16.1.128/28 | 172.16.1.129 |
| 100 | SERVERS | 172.16.1.144/28 | 172.16.1.145 |
| 200 | WIRELESS | 172.16.1.0/26 | 172.16.1.1 |
| 999 | NATIVE (unused) | — | — |

## 2. Quick Reference — Point-to-Point Links

| Link | Subnet | IP A | IP B |
| --- | --- | --- | --- |
| Router–ISP | 172.16.1.160/30 | R1 .161 | ISP-RTR .162 |
| Router–FW1 | 172.16.1.164/30 | R1 .165 | FW1 .166 |
| FW1–CoreSW1 | 172.16.1.168/30 | FW1 .169 | CORE-SW1 .170 |
| FW1–CoreSW2 | 172.16.1.172/30 | FW1 .173 | CORE-SW2 .174 |
| Server–CoreSW1 | 172.16.1.176/30 | SRV1 .177 | CORE-SW1 .178 |
| Server–CoreSW2 | 172.16.1.180/30 | SRV1 .181 | CORE-SW2 .182 |
| CoreSW1–CoreSW2 | 172.16.1.184/30 | CORE-SW1 .185 | CORE-SW2 .186 |
| CoreSW1–DistSW1 | 172.16.1.188/30 | CORE-SW1 .189 | DIST-SW1 .190 |
| CoreSW1–DistSW2 | 172.16.1.192/30 | CORE-SW1 .193 | DIST-SW2 .194 |
| CoreSW2–DistSW1 | 172.16.1.196/30 | CORE-SW2 .197 | DIST-SW1 .198 |
| CoreSW2–DistSW2 | 172.16.1.200/30 | CORE-SW2 .201 | DIST-SW2 .202 |
| DistSW1–DistSW2 | 172.16.1.204/30 | DIST-SW1 .205 | DIST-SW2 .206 |
| WLC–CoreSW1 | 172.16.1.208/30 | WLC1 .209 | CORE-SW1 .210 |

## 3. ROUTER — R1

Role: Edge device connecting the university to the ISP router and passing traffic into FW1.

Design note: the original topology used a Packet Tracer “Cloud” object to simulate the internet. During testing, this was replaced with a real ISP router (see Section 4a) for more realistic, testable NAT verification — pinging a simulated public address (8.8.8.8) instead of an abstract cloud object.

enable
configure terminal
hostname R1

! Interface facing the ISP Router (simulating the internet)
interface GigabitEthernet0/0/0
 description Link_to_ISP
 ip address 172.16.1.161 255.255.255.252
 no shutdown

! Interface facing FW1
interface GigabitEthernet0/0/1
 description Link_to_FW1
 ip address 172.16.1.165 255.255.255.252
 no shutdown

! Default route out to the ISP
ip route 0.0.0.0 0.0.0.0 172.16.1.162

! Centralized RADIUS authentication (see Section 14) with local fallback
aaa new-model
radius-server host 172.16.1.177 key RadiusKey123
aaa authentication login default group radius local

! Basic hardening
service password-encryption
enable secret Cisco123!
username admin privilege 15 secret Cisco123!
line console 0
 password Cisco123!
 login
line vty 0 4
 login local
 transport input ssh

! Loopback for stable RADIUS source (see Section 14)
interface Loopback0
 ip address 172.16.1.211 255.255.255.255
router ospf 1
 network 172.16.1.211 0.0.0.0 area 0

end
write memory

## 4a. ISP ROUTER — ISP-RTR (replaces the Cloud object)

Role: Simulates the public internet — gives R1 something real to route to and get replies from, so NAT can be tested end-to-end (e.g., ping 8.8.8.8).

enable
configure terminal
hostname ISP-RTR

interface GigabitEthernet0/0/0
 description Link_to_R1
 ip address 172.16.1.162 255.255.255.252
 no shutdown

! Loopback simulating a public internet address for testing
interface Loopback0
 ip address 8.8.8.8 255.255.255.255

! Route back toward the university network (via R1)
ip route 0.0.0.0 0.0.0.0 172.16.1.161

end
write memory

## 4. FIREWALL / NAT ROUTER — FW1

Role: Filters and NATs traffic between R1 and the internal network (dual-homed to both Core switches for redundancy).

Design note — important platform finding: the original design used a dedicated Cisco ASA-style firewall device (ISA 3000 model in Packet Tracer). During testing, this device was found to be factory-locked into “transparent mode”, which only bridges traffic between interfaces on the same subnet — it cannot route between the differently-addressed subnets this design requires, and Packet Tracer provides no CLI or GUI option to change this on that specific model. After confirming no alternative Firewall model was available, FW1 was rebuilt as a standard router running NAT and ACLs — a completely standard, real-world-valid approach (many smaller networks use a router for exactly this role instead of a dedicated firewall appliance). This preserves the original IP addressing on the CoreSW1/CoreSW2-facing interfaces, so no changes were needed on either Core switch.

enable
configure terminal
hostname FW1

! Interface facing R1 (outside)
interface GigabitEthernet0/0/0
 description Link_to_R1
 ip address 172.16.1.166 255.255.255.252
 ip nat outside
 no shutdown

! Interface facing CORE-SW1 (inside)
interface GigabitEthernet0/0/2
 description Link_to_CoreSW1
 ip address 172.16.1.169 255.255.255.252
 ip nat inside
 no shutdown

! Interface facing CORE-SW2 (inside)
interface GigabitEthernet0/0/3
 description Link_to_CoreSW2
 ip address 172.16.1.173 255.255.255.252
 ip nat inside
 no shutdown

! NAT: translate the whole internal address space out through the outside interface (PAT/overload)
access-list 1 permit 172.16.0.0 0.0.255.255
ip nat inside source list 1 interface GigabitEthernet0/0/0 overload

! Default route out to R1
ip route 0.0.0.0 0.0.0.0 172.16.1.165

! Routes back into the internal network (CoreSW1 primary, CoreSW2 as floating backup)
ip route 172.16.0.0 255.255.0.0 172.16.1.170
ip route 172.16.0.0 255.255.0.0 172.16.1.174 5

! Basic hardening
service password-encryption
enable secret Cisco123!
username admin privilege 15 secret Cisco123!
line vty 0 4
 login local
 transport input ssh

end
write memory

Verified working in Packet Tracer: - show ip nat translations on FW1 showed live translations from internal PC addresses to FW1’s outside IP - PC ping to 8.8.8.8 (the simulated public address on ISP-RTR) succeeded, confirming the full path: LAN → FW1 (NAT) → R1 → ISP-RTR → reply back

## 5. CORE SWITCH 1 — CORE-SW1

Role: Layer-3 routing hub. No VLANs/SVIs here — only routed point-to-point interfaces + OSPF.

enable
configure terminal
hostname CORE-SW1
ip routing

interface GigabitEthernet1/1/4
 description Link_to_Firewall
 no switchport
 ip address 172.16.1.170 255.255.255.252
 no shutdown

interface GigabitEthernet1/0/1
 description Link_to_Server_NIC1
 no switchport
 ip address 172.16.1.178 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/1
 description Link_to_CoreSW2
 no switchport
 ip address 172.16.1.185 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/2
 description Link_to_DistSW1
 no switchport
 ip address 172.16.1.189 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/3
 description Link_to_DistSW2
 no switchport
 ip address 172.16.1.193 255.255.255.252
 no shutdown

interface GigabitEthernet1/0/2
 description Link_to_WLC1
 no switchport
 ip address 172.16.1.210 255.255.255.252
 no shutdown

! OSPF routing — advertise all connected routed links
router ospf 1
 network 172.16.1.168 0.0.0.3 area 0
 network 172.16.1.176 0.0.0.3 area 0
 network 172.16.1.184 0.0.0.3 area 0
 network 172.16.1.188 0.0.0.3 area 0
 network 172.16.1.192 0.0.0.3 area 0
 network 172.16.1.208 0.0.0.3 area 0
 default-information originate

service password-encryption
enable secret Cisco123!

end
write memory

## 6. CORE SWITCH 2 — CORE-SW2

enable
configure terminal
hostname CORE-SW2
ip routing

interface GigabitEthernet1/1/4
 description Link_to_Firewall
 no switchport
 ip address 172.16.1.174 255.255.255.252
 no shutdown

interface GigabitEthernet1/0/1
 description Link_to_Server_NIC2
 no switchport
 ip address 172.16.1.182 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/1
 description Link_to_CoreSW1
 no switchport
 ip address 172.16.1.186 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/2
 description Link_to_DistSW2
 no switchport
 ip address 172.16.1.201 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/3
 description Link_to_DistSW1
 no switchport
 ip address 172.16.1.197 255.255.255.252
 no shutdown

router ospf 1
 network 172.16.1.172 0.0.0.3 area 0
 network 172.16.1.180 0.0.0.3 area 0
 network 172.16.1.184 0.0.0.3 area 0
 network 172.16.1.196 0.0.0.3 area 0
 network 172.16.1.200 0.0.0.3 area 0

service password-encryption
enable secret Cisco123!

end
write memory

## 7. DISTRIBUTION SWITCH 1 — DIST-SW1

Role: This is where VLAN gateways (SVIs) live for the VLANs facing its access switches, DHCP relay, trunk ports down to Access, and routed uplinks to Core.

(Assuming DIST-SW1 serves ACS1–ACS10 alongside DIST-SW2 for redundancy — every department VLAN’s SVI + DHCP relay should exist on BOTH distribution switches using HSRP/VRRP in a full production build; for a class project, configuring the SVIs on both with matching virtual gateway or simply on DIST-SW1 as primary is acceptable — confirm with your teacher which model they expect.)

enable
configure terminal
hostname DIST-SW1
ip routing

! ---- VLAN Database ----
vlan 10
 name INFORMATICS
vlan 20
 name ENGINEERING
vlan 30
 name LIBRARY
vlan 40
 name ADMIN
vlan 50
 name STAFF
vlan 99
 name MGMT
vlan 100
 name SERVERS
vlan 200
 name WIRELESS
vlan 999
 name NATIVE_UNUSED

! ---- SVIs (VLAN Gateways) with HSRP ----
! DIST-SW1 = HSRP ACTIVE (higher priority) for all 6 user/wireless VLANs
interface vlan 10
 ip address 172.16.0.2 255.255.255.192
 ip helper-address 172.16.1.177
 standby 10 ip 172.16.0.1
 standby 10 priority 110
 standby 10 preempt
 no shutdown
interface vlan 20
 ip address 172.16.0.66 255.255.255.192
 ip helper-address 172.16.1.177
 standby 20 ip 172.16.0.65
 standby 20 priority 110
 standby 20 preempt
 no shutdown
interface vlan 30
 ip address 172.16.0.130 255.255.255.192
 ip helper-address 172.16.1.177
 standby 30 ip 172.16.0.129
 standby 30 priority 110
 standby 30 preempt
 no shutdown
interface vlan 40
 ip address 172.16.0.194 255.255.255.192
 ip helper-address 172.16.1.177
 standby 40 ip 172.16.0.193
 standby 40 priority 110
 standby 40 preempt
 no shutdown
interface vlan 50
 ip address 172.16.1.66 255.255.255.192
 ip helper-address 172.16.1.177
 standby 50 ip 172.16.1.65
 standby 50 priority 110
 standby 50 preempt
 no shutdown
! VLAN 99 (Management) and VLAN 100 (Servers) stay simple — single SVI on DIST-SW1 only (no HSRP, Option B)
interface vlan 99
 ip address 172.16.1.129 255.255.255.240
 no shutdown
interface vlan 100
 ip address 172.16.1.145 255.255.255.240
 no shutdown
interface vlan 200
 ip address 172.16.1.2 255.255.255.192
 ip helper-address 172.16.1.177
 standby 200 ip 172.16.1.1
 standby 200 priority 110
 standby 200 preempt
 no shutdown

! ---- Trunk ports down to Access switches ----
interface range GigabitEthernet1/0/1 - 10
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,99,100,200

! ---- Routed uplinks to Core (no switchport) ----
interface GigabitEthernet1/1/1
 description Link_to_CoreSW1
 no switchport
 ip address 172.16.1.190 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/3
 description Link_to_CoreSW2
 no switchport
 ip address 172.16.1.198 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/2
 description Link_to_DistSW2
 no switchport
 ip address 172.16.1.205 255.255.255.252
 no shutdown

! ---- OSPF ----
router ospf 1
 network 172.16.0.0 0.0.0.63 area 0
 network 172.16.0.64 0.0.0.63 area 0
 network 172.16.0.128 0.0.0.63 area 0
 network 172.16.0.192 0.0.0.63 area 0
 network 172.16.1.64 0.0.0.63 area 0
 network 172.16.1.0 0.0.0.63 area 0
 network 172.16.1.128 0.0.0.15 area 0
 network 172.16.1.144 0.0.0.15 area 0
 network 172.16.1.188 0.0.0.3 area 0
 network 172.16.1.196 0.0.0.3 area 0
 network 172.16.1.204 0.0.0.3 area 0

service password-encryption
enable secret Cisco123!

end
write memory

## 8. DISTRIBUTION SWITCH 2 — DIST-SW2

(Same VLAN database, mirrors DIST-SW1’s role — configure identically for VLANs/trunks, only the routed uplink IPs differ.)

enable
configure terminal
hostname DIST-SW2
ip routing

vlan 10
 name INFORMATICS
vlan 20
 name ENGINEERING
vlan 30
 name LIBRARY
vlan 40
 name ADMIN
vlan 50
 name STAFF
vlan 99
 name MGMT
vlan 100
 name SERVERS
vlan 200
 name WIRELESS
vlan 999
 name NATIVE_UNUSED

! ---- SVIs (VLAN Gateways) with HSRP ----
! DIST-SW2 = HSRP STANDBY (lower priority) for all 6 user/wireless VLANs
! VLAN 99 (Management) and VLAN 100 (Servers) are NOT configured here — they stay single-homed on DIST-SW1 only (Option B)
interface vlan 10
 ip address 172.16.0.3 255.255.255.192
 ip helper-address 172.16.1.177
 standby 10 ip 172.16.0.1
 standby 10 priority 90
 no shutdown
interface vlan 20
 ip address 172.16.0.67 255.255.255.192
 ip helper-address 172.16.1.177
 standby 20 ip 172.16.0.65
 standby 20 priority 90
 no shutdown
interface vlan 30
 ip address 172.16.0.131 255.255.255.192
 ip helper-address 172.16.1.177
 standby 30 ip 172.16.0.129
 standby 30 priority 90
 no shutdown
interface vlan 40
 ip address 172.16.0.195 255.255.255.192
 ip helper-address 172.16.1.177
 standby 40 ip 172.16.0.193
 standby 40 priority 90
 no shutdown
interface vlan 50
 ip address 172.16.1.67 255.255.255.192
 ip helper-address 172.16.1.177
 standby 50 ip 172.16.1.65
 standby 50 priority 90
 no shutdown
interface vlan 200
 ip address 172.16.1.3 255.255.255.192
 ip helper-address 172.16.1.177
 standby 200 ip 172.16.1.1
 standby 200 priority 90
 no shutdown

! Trunk ports down to Access switches
interface range GigabitEthernet1/0/1 - 10
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
 switchport trunk allowed vlan 10,20,30,40,50,99,100,200

! Routed uplinks to Core
interface GigabitEthernet1/1/3
 description Link_to_CoreSW2
 no switchport
 ip address 172.16.1.202 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/1
 description Link_to_CoreSW1
 no switchport
 ip address 172.16.1.194 255.255.255.252
 no shutdown

interface GigabitEthernet1/1/2
 description Link_to_DistSW1
 no switchport
 ip address 172.16.1.206 255.255.255.252
 no shutdown

router ospf 1
 network 172.16.0.0 0.0.0.63 area 0
 network 172.16.0.64 0.0.0.63 area 0
 network 172.16.0.128 0.0.0.63 area 0
 network 172.16.0.192 0.0.0.63 area 0
 network 172.16.1.64 0.0.0.63 area 0
 network 172.16.1.0 0.0.0.63 area 0
 network 172.16.1.192 0.0.0.3 area 0
 network 172.16.1.200 0.0.0.3 area 0
 network 172.16.1.204 0.0.0.3 area 0

service password-encryption
enable secret Cisco123!

end
write memory

Note on redundancy: In this simplified school-project version, VLAN SVIs are active on DIST-SW1 only (DIST-SW2 just trunks and routes). If your teacher expects true first-hop redundancy, add HSRP on both distribution switches for each VLAN — ask if this is required before your final submission, since it adds meaningful extra configuration.

## 9. ACCESS SWITCHES — Template (ACS1 + ACS2 shown, repeat pattern for ACS3–ACS10)

Role: Connect end-user PCs (access ports) and trunk up to both Distribution switches for redundancy. Port security applied on every user-facing port.

### ACS1 (VLAN 10 – Informatics)

enable
configure terminal
hostname ACS1

vlan 10
 name INFORMATICS
vlan 99
 name MGMT
vlan 999
 name NATIVE_UNUSED

! Management IP for remote access (SSH into the switch itself)
interface vlan 99
 ip address 172.16.1.131 255.255.255.240
 no shutdown
ip default-gateway 172.16.1.129

! Trunk ports UP to both Distribution switches
interface FastEthernet0/1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
interface FastEthernet0/2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999

! Access ports DOWN to PCs (VLAN 10) — Fa0/23 excluded, reserved for AP1 (see Section 12 below)
interface range FastEthernet0/3 - 22
 switchport mode access
 switchport access vlan 10
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation shutdown
 switchport port-security mac-address sticky
 no shutdown

service password-encryption
enable secret Cisco123!

end
write memory

### ACS2 (VLAN 10 – Informatics, same floor/dept as ACS1)

enable
configure terminal
hostname ACS2

vlan 10
 name INFORMATICS
vlan 99
 name MGMT
vlan 999
 name NATIVE_UNUSED

interface vlan 99
 ip address 172.16.1.132 255.255.255.240
 no shutdown
ip default-gateway 172.16.1.129

interface FastEthernet0/1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999
interface FastEthernet0/2
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk native vlan 999

interface range FastEthernet0/3 - 24
 switchport mode access
 switchport access vlan 10
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation shutdown
 switchport port-security mac-address sticky
 no shutdown

service password-encryption
enable secret Cisco123!

end
write memory

### Repeat this exact template for the rest — only change: hostname, the VLAN number, and the management IP’s last octet (.133, .134 … up to .140)

| Switch | VLAN (access vlan #) | Mgmt IP (VLAN 99) | Note |
| --- | --- | --- | --- |
| ACS3 | 20 (Engineering) | 172.16.1.133 | Exclude Fa0/23 from PC range — reserved for AP2 |
| ACS4 | 20 (Engineering) | 172.16.1.134 | Full Fa0/3–24 available for PCs |
| ACS5 | 30 (Library) | 172.16.1.135 | Exclude Fa0/20 from PC range — reserved for AP3 |
| ACS6 | 30 (Library) | 172.16.1.136 | Full Fa0/3–24 available for PCs |
| ACS7 | 40 (Admin) | 172.16.1.137 | Exclude Fa0/19 from PC range — reserved for AP4 |
| ACS8 | 40 (Admin) | 172.16.1.138 | Full Fa0/3–24 available for PCs |
| ACS9 | 50 (Staff) | 172.16.1.139 | Exclude Fa0/19 from PC range — reserved for AP5 |
| ACS10 | 50 (Staff) | 172.16.1.140 | Full Fa0/3–24 available for PCs |

Note: management VLAN 99 pool .129–.142 easily covers all switch management IPs across the whole project (Core, Dist, Access = 14 devices max, we have exactly 14 usable addresses in that /28 — tight but sufficient. If you add more devices later, this pool may need to expand).

AP-facing ports: Since AP1–AP5 connect to specific ports on ACS1, ACS3, ACS5, ACS7, ACS9, use the exact ports below (each switch needs this block in addition to its normal PC access ports and two uplink trunk ports):

| Switch | AP Port |
| --- | --- |
| ACS1 | Fa0/23 |
| ACS3 | Fa0/23 |
| ACS5 | Fa0/20 |
| ACS7 | Fa0/19 |
| ACS9 | Fa0/19 |

! Example for ACS1 (repeat with the matching port number on ACS3, ACS5, ACS7, ACS9)
interface FastEthernet0/23
 switchport mode access
 switchport access vlan 200
 no shutdown

Important — do NOT apply port security on AP-facing ports. Unlike a PC port (which only ever carries one device’s MAC address), an AP-facing port carries a different MAC address for every wireless client that connects through that AP over time. Port security was originally applied here during initial configuration (max 1, later raised to max 10) — both were found through live testing to eventually trip into Secure-shutdown as more wireless devices connected, since sticky-learned addresses never expire (Aging Time: 0, disabled) and simply accumulate until the limit is hit again. The correct real-world practice is to not apply port security to AP-facing ports at all — wireless client security is already enforced at the AP/WLC via WPA2 authentication, not by counting MAC addresses on the switch port. See Section 13 (Troubleshooting Log) for the full diagnosis.

The even-numbered switches (ACS2, ACS4, ACS6, ACS8, ACS10) do not need this — they only carry department PC traffic.

## 10. SERVER — SRV1 (DHCP / DNS / Authentication)

Role: Provides DHCP, DNS, and login authentication for the whole network. Dual-homed to both Core switches.

Interface IPs (configured in the Server’s Desktop > IP Configuration, or via CLI if it’s a router-based “server”):

NIC1 (Gi1):  IP 172.16.1.177   Subnet 255.255.255.252   Gateway 172.16.1.178
NIC2 (Fa0):  IP 172.16.1.181   Subnet 255.255.255.252   Gateway 172.16.1.182

DHCP Pool setup (done via Server GUI in Packet Tracer, “Services > DHCP” tab) — one pool per department VLAN:

(Start ranges below skip the first 3 addresses of each subnet: .x = HSRP virtual gateway, .x+1 = DIST-SW1 real IP, .x+2 = DIST-SW2 real IP — see Section 7/8 HSRP config.)

| Pool Name | Network | Default Gateway | Start Range | Max Users |
| --- | --- | --- | --- | --- |
| POOL-INFORMATICS | 172.16.0.0/26 | 172.16.0.1 | 172.16.0.4 | 58 |
| POOL-ENGINEERING | 172.16.0.64/26 | 172.16.0.65 | 172.16.0.68 | 58 |
| POOL-LIBRARY | 172.16.0.128/26 | 172.16.0.129 | 172.16.0.132 | 58 |
| POOL-ADMIN | 172.16.0.192/26 | 172.16.0.193 | 172.16.0.196 | 58 |
| POOL-STAFF | 172.16.1.64/26 | 172.16.1.65 | 172.16.1.68 | 58 |
| POOL-WIRELESS | 172.16.1.0/26 | 172.16.1.1 | 172.16.1.4 | 58 |

DNS setup (Server GUI, “Services > DNS”): add A-records for anything students need to test in Packet Tracer, e.g.:

www.hu-university.local  →  172.16.1.177

AAA/Authentication (simplified for Packet Tracer): configure local usernames on switches/router for login authentication:

username admin privilege 15 secret Cisco123!
line vty 0 4
 login local
 transport input ssh

(For a fuller AAA setup you’d point devices to a RADIUS/TACACS+ server — mention this as a possible future enhancement in your report if your teacher wants it, but local auth is normal for a Packet Tracer-scale project.)

Known limitation — verified through failure testing: the Server is physically dual-homed (one NIC to CoreSW1, one to CoreSW2), and was intended to provide redundancy the same way the Firewall does. However, Packet Tracer’s Server-PT model only supports a single active default gateway at a time — it cannot automatically fail over to its second NIC if the first NIC’s link goes down. This was confirmed by directly testing it: powering off CORE-SW1 made the Server completely unreachable, even though its second cable to CoreSW2 was fully intact and functional (confirmed by the fact that powering off CoreSW2 instead had no effect, since the Server was never routing through it in the first place). The Core↔Distribution mesh itself was separately confirmed to reroute correctly during the same test (see Section 13, entries 11–12) — this limitation is specific to the Server device, not the network fabric. A correct real-world fix would connect the Server through a single access-layer switch with its own dual uplinks to both Distribution switches (the same proven HSRP pattern used for all department VLANs), rather than giving the Server itself two direct Core connections — this was identified as the right solution but not implemented, to avoid further reworking the topology this late in the project.

## 11. WIRELESS LAN CONTROLLER — WLC1

Role: Intended to centrally manage all 5 Access Points; connects via its GigabitEthernet0 port to CoreSW1’s GigabitEthernet1/0/2.

Management IP: 172.16.1.209
Subnet: 255.255.255.252
Default Gateway: 172.16.1.210

WLAN Name/SSID: HU-WIFI
VLAN: 200 (Wireless)
Authentication: WPA2-PSK
Passphrase: HUWifi2026!
Encryption: AES
Central Control: Local switching, local authentication

Implementation note (found during Packet Tracer testing): the standard “Access Point-PT” model used in this project is a standalone AP — it broadcasts its own SSID independently and does not actually associate with or receive configuration from a WLC in Packet Tracer’s simulation (unlike real lightweight APs, which rely entirely on a controller via CAPWAP). Because of this, each AP below was configured individually with matching SSID/security settings so they behave as one consistent network to users. The WLC remains part of the design as the intended centralized management point (a valid, standard real-world architecture worth describing in your report), with this simulation limitation noted as a known constraint of the Packet Tracer AP model used.

Known limitation — single point of failure: the WLC device model used in this project has only one physical Ethernet port, so it can only connect to CoreSW1 — it cannot be dual-homed to both Core switches like the Firewall and Access switches are (the Server is also dual-cabled, but as found and confirmed through failure testing below, its redundancy does not actually function — see Section 10). If CoreSW1 fails, the WLC becomes unreachable. In this specific implementation the practical impact is minimal, since (per the note above) the APs continue broadcasting and serving wireless clients independently of the WLC’s availability — only centralized management would be temporarily lost, not actual wireless service. A real-world fix would be to connect the WLC through an intermediary access switch with its own dual uplinks to both Core switches, giving the WLC indirect redundancy; this was considered but not implemented, to keep the topology from growing further at this stage of the project.

Access Points (AP1–AP5): each configured individually with identical settings so users see one seamless “HU-WIFI” network across every floor:

SSID: HU-WIFI
Authentication: WPA2-PSK
Passphrase: HUWifi2026!
Encryption: AES

| AP | Location | Connects to (access port, VLAN 200) | 2.4GHz Channel |
| --- | --- | --- | --- |
| AP1 | Building-A, Floor 1 | ACS1, Fa0/23 | 1 |
| AP2 | Building-A, Floor 2 | ACS3, Fa0/23 | 6 |
| AP3 | Building-B, Floor 1 | ACS5, Fa0/20 | 11 |
| AP4 | Building-B, Floor 2 | ACS7, Fa0/19 | 1 |
| AP5 | Building-B, Floor 3 | ACS9, Fa0/19 | 6 |

(Channels 1/6/11 are the standard non-overlapping 2.4GHz channels — alternating them between nearby/adjacent floors reduces interference.)

## 12. Security Summary Applied Across the Network

- Port security on every PC-facing access port (max 2 MACs, sticky, violation = shutdown)

- Native VLAN changed to 999 (unused) on every trunk — standard best practice against VLAN-hopping attacks

- Management VLAN 99 isolated — only reachable for switch/router administration, never assigned to end users

- ACL on Firewall blocking unsolicited inbound traffic from the Internet

- SSH-only remote management (Telnet disabled) with local login credentials

- OSPF provides automatic failover across all redundant links (Core↔Core, Core↔Dist, Dist↔Dist)

## 13. Troubleshooting Log — Issues Found & Fixed During Testing

This section documents real issues discovered while implementing this design in Packet Tracer, and how they were resolved. Including this in your report demonstrates the “simulation and testing” requirement, not just static configuration.

| # | Issue | Root Cause | Fix |
| --- | --- | --- | --- |
| 1 | VLAN 100 SVI had wrong IP (172.16.1.130) | Typo — didn’t match the planned Server subnet 172.16.1.144/28 | Corrected to 172.16.1.145 |
| 2 | OSPF wasn’t advertising the Server VLAN | Missing network 172.16.1.144 0.0.0.15 area 0 statement on DIST-SW1 | Added the missing OSPF network line |
| 3 | Accidental VLAN interfaces created on CORE-SW1 | Config typed on the wrong device — Core switches should have NO VLANs, only routed links | Removed with no interface vlan X for each |
| 4 | ip helper-address and DNS pointed to an IP that didn’t exist anywhere (172.16.1.146) | VLAN 100 was originally planned to host the Server, but the final design connected the Server via dedicated point-to-point links instead — the .146 address was never actually assigned to any device | Replaced every instance of 172.16.1.146 with the Server’s real, reachable IP 172.16.1.177 |
| 5 | Server’s two NICs were physically swapped vs. the planned IP scheme | FastEthernet0 was cabled to Core SW2, GigabitEthernet1 to Core SW1 — opposite of the original plan | Swapped the IP addresses on each interface to match the real cabling, rather than re-cabling |
| 6 | DHCP still failed even after routing was confirmed working (ping succeeded end-to-end) | Packet Tracer’s Server-PT binds DHCP pools to whichever interface was selected when the pool was created — all 6 pools were built under FastEthernet0, but the server’s actual working/routable interface was GigabitEthernet1 | Re-created all 6 DHCP pools with the Interface dropdown set to GigabitEthernet1 |
| 7 | RADIUS/AAA commands (radius-server host, aaa new-model) returned “Unrecognized command” on every switch | The switch IOS images used in Packet Tracer for this project’s Catalyst-style Layer-3 switches do not implement RADIUS/AAA at all — a genuine platform limitation, not a config error | Limited centralized RADIUS authentication to the Router (R1) only; all switches keep local username/password authentication (already fully functional) |
| 8 | Traffic could not pass through the Firewall in either direction, even though every routing/OSPF check on both sides was correct (0% ping success through FW1’s interfaces, show xlate showed 0 translations) | The ASA-style Firewall device (ISA 3000 model) was factory-locked into “firewall transparent” mode, which only bridges traffic between interfaces on the same subnet — it cannot route between differently-addressed subnets, which this design requires. No CLI (no firewall transparent) or GUI option existed to change this on the available device model | Rebuilt FW1 as a standard router running NAT (ip nat inside/outside + ip nat inside source list ... overload) and static routing instead of the ASA — a standard, real-world-valid alternative. Reused the exact same IP addresses on the CoreSW1/CoreSW2-facing interfaces, so no changes were needed on either Core switch |
| 9 | No default route existed anywhere pointing toward the Firewall/internet | default-information originate was configured on CORE-SW1’s OSPF process, but no actual ip route 0.0.0.0 0.0.0.0 <next-hop> static default route existed for it to advertise — so nothing was ever actually distributed | Added real static default routes on FW1 pointing to R1, and confirmed via show ip route that “Gateway of last resort” correctly propagated all the way down to DIST-SW1 via OSPF |
| 10 | Packet Tracer’s Cloud object made NAT difficult to verify (no real address to ping) | The Cloud-PT object doesn’t provide a meaningful, pingable public IP for end-to-end NAT testing | Replaced the Cloud with a real ISP router (ISP-RTR) with a loopback address (8.8.8.8) simulating a public internet host, enabling a genuine, verifiable NAT test (ping 8.8.8.8 from an internal PC) |
| 11 | Redundancy failure test: powering off CORE-SW1 caused the Server to become completely unreachable | The Server’s two NICs sit in two separate, independent point-to-point subnets, each with only ONE possible path. More fundamentally, Packet Tracer’s Server-PT model only supports a single active default gateway — it cannot automatically fail over from one NIC to the other, even with both physically connected and correctly addressed | Not fixed in this iteration (see Section 10) — identified the correct fix (route the Server through an access-layer switch with HSRP-backed dual uplinks, matching the working VLAN pattern) but chose to document this as a known, deliberate limitation rather than further rework the topology |
| 12 | Redundancy verification test: powering off CORE-SW1 while checking show ip route on DIST-SW1 for a genuinely mesh-connected destination (CoreSW2–DistSW2 link) | N/A — this was a confirmation test, not a bug | Confirmed 8 valid equal-cost alternate paths remained available (via Gi1/1/2, Gi1/1/3, and all 6 VLAN trunks), none through the failed CORE-SW1 — proving the Core↔Distribution mesh redundancy design genuinely works, isolating the Server issue (row 11) as a Server-specific limitation rather than a network-wide fault |
| 13 | Wireless clients intermittently failed to get a DHCP address (self-assigned 169.254.x.x), and the AP-facing access port would go into Secure-shutdown (link light red) after a second wireless device connected | Port security was applied to AP-facing ports with maximum 1 — but an AP-facing port carries a different MAC address for every wireless client that connects through it over time, not just one device like a PC port. Raising the limit to maximum 10 only delayed the same failure (show port-security interface confirmed Total MAC Addresses: 10, with sticky-learned addresses never expiring since Aging Time: 0 is disabled) | Removed port security entirely from all 5 AP-facing ports (ACS1 Fa0/23, ACS3 Fa0/23, ACS5 Fa0/20, ACS7 Fa0/19, ACS9 Fa0/19) — the correct real-world practice, since wireless client security is already enforced at the AP via WPA2 authentication, not by counting MAC addresses on the switch port |

Verification performed: - ping tests from Core SW1 and Distribution SW1 to the Server (100% success after fixes) - show ip ospf neighbor confirmed FULL adjacencies across Core/Distribution - show ip route confirmed OSPF-learned routes to every subnet, including the default route - Packet Tracer Simulation Mode used to trace the actual DHCP packet path and confirm delivery - PC successfully received a DHCP-assigned IP, gateway, and DNS server - PC successfully resolved www.hu-university.local and pinged it successfully - show ip nat translations on FW1 confirmed live NAT translations - PC successfully pinged 8.8.8.8 (ISP-RTR’s simulated public address), confirming full NAT round-trip - Live failure test: powered off CORE-SW1 entirely — Core↔Distribution mesh correctly rerouted (8 alternate paths confirmed via show ip route); Server-specific redundancy confirmed non-functional (see rows 11–12); network fully recovered and reconverged after powering CORE-SW1 back on - show port-security interface used to directly diagnose the AP port shutdown issue (row 13), confirming the exact cause before removing port security from those ports

## 14. Centralized Authentication (RADIUS / AAA)

Why: instead of every device checking its own local username list, devices ask the Server “is this login valid?” — one place to manage credentials, satisfying the “authentication” requirement from the project brief (alongside DHCP and DNS).

Important finding from Packet Tracer testing: the switch IOS images used in this project (Catalyst-style Layer-3 switches) do not support RADIUS/AAA commands at all (radius-server, radius server, aaa new-model referencing RADIUS all return “Unrecognized command” or errors on switches). This is a genuine limitation of the simulated switch IOS, not a configuration mistake. Only the Router (R1) supports and runs centralized RADIUS authentication. All switches (Core, Distribution, Access) continue using local username/password authentication, which is fully functional and was already configured in Sections 3–9. This is documented here as a tested, real finding — not every design decision survives contact with the simulation tooling, and adapting to platform constraints is part of real network engineering.

### Step A — Set up the RADIUS server (on SRV1)

- Click Server0 → Services tab → AAA

- Turn Service: On (Radius Port stays default: 1645)

- Under Network Configuration, register R1 as a client:

| Client Name | Client IP | Secret | ServerType |
| --- | --- | --- | --- |
| R1 | 172.16.1.211 | RadiusKey123 | Radius |

(Additional clients for switches were tested and added but have no effect, since the switches themselves cannot make RADIUS requests — they are harmless to leave registered on the server, or can be removed for a cleaner report.)

- Under User Setup, add the account RADIUS will validate:

Username: admin
Password: Cisco123!

### Step B — Give R1 a stable Loopback IP (source of its RADIUS requests)

interface Loopback0
 ip address 172.16.1.211 255.255.255.255
exit
router ospf 1
 network 172.16.1.211 0.0.0.0 area 0

### Step C — Configure R1 for centralized RADIUS with local fallback

aaa new-model
radius-server host 172.16.1.177 key RadiusKey123
aaa authentication login default group radius local
username admin privilege 15 secret Cisco123!

How this works: R1 tries RADIUS first — if the Server is unreachable, it automatically falls back to the local username/password (admin/Cisco123!). This is a safe, standard design — you’re never locked out even during a network fault.

Verified working — confirmed via:

show run | section radius
show run | include aaa

Output showed the RADIUS server IP (172.16.1.177), correct auth port (1645), matching key (RadiusKey123), and aaa authentication login default group radius local correctly applied.

### All switches (Core, Distribution, Access): local authentication only

Already configured in each device’s section above:

username admin privilege 15 secret Cisco123!
line vty 0 4
 login local
 transport input ssh

This remains the authentication method for every switch in this design, and is fully sufficient for device management security at this network’s scale.

## 15. Access Control Lists (ACLs) — Tested Configuration

Applied on both DIST-SW1 and DIST-SW2, matching traffic entering each VLAN’s SVI.

Note: Cisco IOS reserves ACL numbers 1–99 for Standard ACLs and 100–199 for Extended ACLs (which match both source AND destination — what we need here). Using “200” causes an “Invalid input” error, since it falls outside the extended range — 101 is used instead for the Wireless ACL.

### ACL 100 — Applied to VLANs 10, 20, 30, 40, 50 (Departments)

access-list 100 permit ip 172.16.0.0 0.0.0.63 172.16.1.144 0.0.0.15
access-list 100 permit ip 172.16.0.64 0.0.0.63 172.16.1.144 0.0.0.15
access-list 100 permit ip 172.16.0.128 0.0.0.63 172.16.1.144 0.0.0.15
access-list 100 permit ip 172.16.0.192 0.0.0.63 172.16.1.144 0.0.0.15
access-list 100 permit ip 172.16.1.64 0.0.0.63 172.16.1.144 0.0.0.15
access-list 100 deny ip any 172.16.1.128 0.0.0.15
access-list 100 permit ip any any

Logic: allow all 5 departments to reach the Server subnet → block everyone from reaching Management → allow everything else (Internet, inter-department traffic).

### ACL 101 — Applied to VLAN 200 (Wireless)

access-list 101 permit ip 172.16.1.0 0.0.0.63 172.16.1.144 0.0.0.15
access-list 101 deny ip 172.16.1.0 0.0.0.63 172.16.1.128 0.0.0.15
access-list 101 deny ip 172.16.1.0 0.0.0.63 172.16.0.0 0.0.0.63
access-list 101 deny ip 172.16.1.0 0.0.0.63 172.16.0.64 0.0.0.63
access-list 101 deny ip 172.16.1.0 0.0.0.63 172.16.0.128 0.0.0.63
access-list 101 deny ip 172.16.1.0 0.0.0.63 172.16.0.192 0.0.0.63
access-list 101 deny ip 172.16.1.0 0.0.0.63 172.16.1.64 0.0.0.63
access-list 101 permit ip any any

Logic: Wireless can reach the Server → blocked from Management → blocked from every wired department VLAN (extra isolation since wireless is less trusted) → everything else (mainly Internet) allowed.

### Applying both ACLs to interfaces

interface vlan 10
 ip access-group 100 in
interface vlan 20
 ip access-group 100 in
interface vlan 30
 ip access-group 100 in
interface vlan 40
 ip access-group 100 in
interface vlan 50
 ip access-group 100 in
interface vlan 200
 ip access-group 101 in

Verified working in Packet Tracer: show access-lists confirmed both ACLs actively matching live traffic (department-to-Server/Internet traffic hitting the final permit line, wireless-to-department traffic correctly being denied).

## 16. NAT (Network Address Translation) — Explained

### Why NAT is needed

Every device in this network uses a private IP address (172.16.0.0/16, an RFC 1918 range). Private addresses are only meaningful inside your network — the public internet will never route traffic to or from them. Without translation, a student’s PC at 172.16.0.5 could never successfully communicate with a public website, because reply traffic would have nowhere valid to go.

NAT solves this by translating private source addresses into a real, routable address as traffic leaves the network via FW1 — and translating replies back to the correct internal device on the way in.

### The type of NAT used in this design: PAT (Port Address Translation)

This project uses dynamic PAT (also called NAT Overload), configured on FW1 using standard Cisco IOS commands:

access-list 1 permit 172.16.0.0 0.0.255.255
ip nat inside source list 1 interface GigabitEthernet0/0/0 overload

| NAT Type | How it works | Used here? |
| --- | --- | --- |
| Static NAT (1-to-1) | One private IP always maps to one dedicated public IP | ❌ Not used — would need hundreds of public IPs |
| Dynamic NAT (pool) | Private IPs map to a pool of public IPs, first-come-first-served | ❌ Not used — still needs multiple public IPs |
| PAT / NAT Overload (many-to-1) | ALL internal devices share one single outside IP, distinguished by port number | ✅ This is what’s configured |

PAT is the standard choice for this kind of network — one outside-facing interface, potentially hundreds of PCs/laptops/APs, all sharing that one address efficiently.

### Step-by-step: what happens to a single packet

Say a PC in the Informatics VLAN (172.16.0.5) opens a browser to visit a website:

| Step | What happens |
| --- | --- |
| 1 | PC sends a packet: Source 172.16.0.5:5000 → Destination <website IP>:80 |
| 2 | Packet routes through ACS1 → DIST-SW1 → CORE-SW1 → arrives at FW1’s inside interface (Gi0/0/2) |
| 3 | FW1’s NAT rule matches (source is inside 172.16.0.0/16, per access-list 1) — it rewrites the source to its own outside interface IP and a new port, e.g. 172.16.1.166:40001 |
| 4 | FW1 remembers this translation in its NAT table: 172.16.0.5:5000 ↔ 172.16.1.166:40001 |
| 5 | Packet continues: FW1 → R1 → ISP-RTR → the destination, which only ever sees FW1’s outside IP — never 172.16.0.5 |
| 6 | Reply comes back addressed to 172.16.1.166:40001 |
| 7 | FW1 checks its NAT table, finds the match, and translates the destination back to 172.16.0.5:5000 |
| 8 | Reply is routed back down to the PC — from the PC’s point of view, it looks like a completely normal, direct conversation |

The PC never knows NAT happened — everything is transparent from its perspective.

### How PAT tells devices apart (this is the “P” in PAT)

Since every internal device shares the same outside IP, FW1 uses the port number as the distinguishing factor:

172.16.0.5:5000   →  172.16.1.166:40001
172.16.0.9:5000   →  172.16.1.166:40002
172.16.1.10:5000  →  172.16.1.166:40003

Even though all three internal PCs might use the same source port (5000) internally, FW1 assigns each one a different outside port, so replies always get routed back to the correct device. This translation table can hold thousands of simultaneous sessions.

### Verifying NAT is working (commands on FW1)

show ip nat translations
show ip nat statistics

- show ip nat translations — shows the live translation table: every currently-active internal-IP-to-outside-port mapping in real time

- show ip nat statistics — shows hit counts and totals, confirming the rule is actively being used

Verified working in Packet Tracer: show ip nat translations showed live entries mapping internal PC addresses to FW1’s outside IP, and a PC successfully pinged 8.8.8.8 (ISP-RTR’s simulated public loopback), confirming the full round-trip: LAN → FW1 (NAT) → R1 → ISP-RTR → reply correctly translated back to the PC.

### What NAT does NOT do

It’s worth being clear about this for your report: NAT is not a security feature by itself, even though it has the side effect of hiding internal IP addressing from the outside world. The actual security enforcement in this design comes from the ACLs (Section 15) — NAT’s job here is purely address translation, not access control.

### End of Configuration Document